Communication Laboratory, Helsinki University of Technology

Hong Zhang (94027T)

GSM 1800 Radio Network Planning

for a Metropolitan Region

S-72.231 Mobile Communication Systems

( The team specific parameter set number is 7 )

LIST

1	Calculation of  maximum cell radius Rmax using maximum base station parameters	3

1.1	The macro cell in the  region A	3

1.1.1	Downlink	3

1.1.2	Uplink	4

1.2	The  region B	4

1.2.1	Downlink	4

1.2.2	Uplink	4

1.3	The region C	5

1.3.1	Downlink	5

1.3.2	Uplink	5

1.4	The micro cell in the region A	5

1.4.1	Downlink	5

1.4.2	Uplink	5

2	Capacity plan	6

2.1	Geometry of the service area	6

2.2	The capacity planning in region A for Macro cell	8

2.3	The capacity planning in region B for Macro cell	9

2.4	The capacity planning in region C for Macro cell	10

2.5	The capacity planning in region A for Micro cell	11

3	Coverage planning	12

3.1	Area A (macro cell)	12

3.1.1	Downlink	12

3.1.2	Uplink	13

3.2	Area B	13

3.2.1	Downlink	13

3.2.2	Uplink	14

3.3	Area C	14

3.3.1	Downlink	14

3.3.2	Uplink	14

3.4	Area A (micro cell)	15

3.4.1	Downlink	15

3.4.2	Uplink	15

4	Frequency planning	17

GSM 1800 Radio Network Plan for a Metrpolitan Region

# Calculation of  maximum cell radius Rmax using maximum base station parameters

antenna gain                     feeder diameter

mast top amplifier            antenna height

signal outage probability :  Fu=90%

Fu=PS(R)+e2(1−ab)b2Q(2b-a)

PS(R)= Q (S−PRX(R)σ)=Q  (−SFMσ)

SMF=INVQ(1-PS(R)).σ

n=4.49-0.655log(hbs)  (macro cell)

hbs= 30m, SFM =3.6 dB

hbs= 120m, SFM =4.2 dB

## The macro cell in the  region A

### Downlink

conditions: antenna gain : Gbs−max =21dBi ,   Gms−max=2dBi

feeder diameter : 2_1/4”     26.3db/km

mast top amplifier : Gmat=10dB , Fmat=2.0dB

antenna height: hbs=30m…..120, hms=1.6m

mobile station sensitivity  Sms: -100dBm

Max base station power levelPbstx: <46dBm

The feeder length of base station is: l=10m+ hbs

hbs= 30m,l=10m+30m=40m

the loss of feeder Lbsf=αl=26.3*18001700*40m=1.1dB

hbs= 120m,l=10m+120m=130m             the loss of feeder Lbsf=αl=3.5dB

the combining filter loss : Ac=3dB

BS connector & jumper losses: 1,0dB

Sms< Pbstx+ Gbs−max-Ac-1.0-Lbsf-Lpath+Gms−max-SMF

hbs= 30m,Lpath <46+21-1.1-1-3+2-3.6-(-100)=160.3dB

Lpath<46.3+33.9log(f)-13.8log(hbs)-A(hms)+(44.9-6.55log(hbs))log(R)+ Cm=160.3

R<4.05km

hbs= 120m,Lpath <46+21-3.5-1-3+2-4.2-(-100)=157.3dB

Lpath<46.3+33.9log(f)-13.8log(hbs)-A(hms)+(44.9-6.55log(hbs))log(R)+ Cm=157.3

R<7.14km

### Uplink

The effective gain of the mast top amplifier is:

Gmtaeff=Gmta-10lg(1+100.1(Gmta+Fmta)−1100.1(αl+Frx))

hbs= 30m             Gmtaeff=6.88dB

Sbs Pmstx+ Gms−max-Lpath-SMF +Gbs−max-Lbsf-1+Gmtaeff

Lpath30+2-3.6+21-1.1-1+6.88-(-102)=156.18Db

Lpath=46.3+33.9log(f)-13.8log(hbs)-A(hms)+(44.9-6.55log(hbs))log(R)+

Cm156.18dB

R3.09 km

hbs= 120m             Gmtaeff=7.95dB

Sbs Pmstx+ Gms−max-Lpath-SMF +Gbs−max-Lbsf-1+Gmtaeff

Lpath30+2-4.2+21-3.5-1+7.95-(-102)=154.25Db

Lpath=46.3+33.9log(f)-13.8log(hbs)-A(hms)+(44.9-6.55log(hbs))log(R)+

Cm154.25dB

R5.71 km

## The region B

### Downlink, using the same method as for region A to calculate the radius

hbs= 30m, R< 10.75km

hbs= 120m, R< 21.45km

### Uplink, using the same method as for region A to calculate the radius

hbs= 30m, R8.21km

hbs= 120m, R 30.44km

## The region C

### Downlink, using the same method as for region A to calculate the radius

hbs= 30m, R< 39.84km

hbs= 120m, R< 93.80km

### 1.3.2	Uplink, using the same method as for region A to calculate the radius

hbs= 30m, R30.44km

hbs= 120m, R 74.94km

## The micro cell in the region A

signal outage probability :  Fu=90%

using COST231 Walfisch-Ikegamipath loss model ,

the path loss exponent n= (20+kd)/10=4.8

the service probability at the cell border Ps(R)=0.68

SFM =0.45*6 dB=2.7dB

### Downlink

The feeder length of base station is: l=2m+ hbs

hbs= 10m,l=12m

the loss of feeder Lbsf=αl=26.3*18001700*12m=0.32dB

Sms< Pbstx+ Gbs−max-Ac-1.0-Lbsf-Lpath+Gms−max-SMF

Lpath <46+15-3-1-0.32+2-2.7-(-100)=155.98dB

Lpath=L0+Lrts+Lmsd<155.98dB             R=d<0.91km

### Uplink

The effective gain of the mast top amplifier is:

Gmtaeff=Gmta-10lg(1+100.1(Gmta+Fmta)−1100.1(αl+Frx))

hbs= 10m             Gmtaeff=6.46dB

Sbs Pmstx+ Gms−max-Lpath-SMF +Gbs−max-Lbsf-1+Gmtaeff

Lpath30+2-2.7+15-0.32-1+6.46-(-102)=151.44dB

Lpath=L0+Lrts+Lmsd151.44dB             R=d0.73km

Summary of the maximum cell radius

|  | Region A (macrocell) | Region B | Region C | Region A(microcell) |
| --- | --- | --- | --- | --- |
| Feeder diameter 2_1/4 | 26.3dB/km | 26.3dB/km | 26.3dB/km | 26.3dB/km |
| Mast top amplifier | Gmat:10dB Fmat:2.0dB | Gmat:10dB Fmat:2.0dB | Gmat:10dB Fmat:2.0dB | Gmat:10dB Fmat:2.0dB |
| BSAntenna height | 30m | 30m | 30m | 10m |
| MSAntenna height | 1.6m | 1.6m | 1.6m | 1.6m |
| BS sensitivity | -102dBm | -102dBm | -102dBm | -102dBm |
| MS sensitivity | -100dBm | -100dBm | -100dBm | -100dBm |
| BS max output power level | <46dBm | <46dBm | <46dBm | <46dBm |
| MS output power level | 30dBm | 30dBm | 30dBm | 30dBm |
| The combining filter loss | 3,0dBm | 3,0dBm | 3,0dBm | 3,0dBm |
| BS connector & jumper loss | 1,0dBm | 1,0dBm | 1,0dBm | 1,0dBm |
| The BS antenna gain | 21dBi | 21dBi | 21dBi | 15dBi |
| The MS antenna gain(including feeder and duplex loss | 2dBi | 2dBi | 2dBi | 2dBi |
|  | hbs=30m |  |  |  |
| The feeder loss | 1.1dB | 1.1dB | 1.1dB | 0.32dB |
| SMF | 3.6dB | 3.6dB | 3.6dB | 2.7dB |
| Loss(path loss+SMF) | 160.3dB | 160.3dB | 160.3dB | 155.98dBm |
| The max cell radius in DL | <4.05km | <10.75km | <39.84km | <0.91km |
|  | hbs=120m without using diversity gain |  |  |  |
| The feeder loss | 3.5dB | 3.5dB | 3.5dB | 0.32dB |
| SMF | 4.2dB | 4.2dB | 4.2dB | 2.7dB |
| The effective mast top amplifier gain | 7.95dB | 7.95dB | 7.95dB | 6.46dB |
| Loss(path loss+SMF) | 154.25dB | 154.25dB | 154.25dB | 151.44dB |
| The max cell radius in UL | 5.71km | 17.14km | 74.94km | 0.73km |

# Capacity plan

## Geometry of the service area

C

B

100km

A

Region Type:

A: city

B: suburban

C: rural

4km

25km

Here using method: Minimum excess capacity with cells extended over region border.

Length of street /road network: L=L12LB1+L22LB2=2ALB

The orginated traffic: Tij=FiCpijToijNij

B=TNN!n=0NTnn!

## The capacity planning in region A for Macro cell

3.15Km

3.15Km

A1

A2

A3

A4

1.15Km

Area A cell size:

16Km2

Region A total area

9.94Km2

x2=                                  =                   =

Minimum cell number

1.61

x=3.15Km              RA=3.15/2=2.23Km

Comparing the radius calculated by maximum base station parameters with this radius in part 1: the latter is smaller, so I select the radius according to capacity.

In the region A, the vehicle traffic is designed in macro cell, the minimum number of BS is 1.61, but there seems no way to design 1.61 cells, I design 4 BSs.

In fact, offered traffic in every cell:

T= 2*2*2.84+(3.15^2-4)*0.14=12.19 Erlang          22 TC/BS giving a capacity of

14.9Erlang /BS, using 3 TRXs/BS

## The capacity planning in region B for Macro cell

B3

A1

A2

A3

A4

B1

B2

B4

14.3Km

14.3Km

1.8Km

1.15Km

Area B cell size:

x2=                                  =                   =

Uncovered area

in region B

Minimum cell number

585.31Km2

2.87

204.03Km2

x=14.3Km              RB=14.3/2=10.1Km

Comparing the radius calculated by maximum base station parameters with this radius in part 1: the latter is smaller, so I select the radius according to capacity.

In the region B, the traffic is designed in macrocell, the minimum number of BS is 2.87, but I design 4 BSs.

In fact, offered traffic in every cell:

T= (12.5^2-3.15^2)*0.14+(14.3^2-12.5^2)*0.02=21.01 Erlang

29 TCs/BS giving a capacity of 21.04Erlang /BS, using 4 TRXs/BS

## The capacity planning in region C for Macro cell

C3

35.7km

28.6km

C1

B3

A1

A2

A3

A4

B1

B2

B4

14.3Km

14.3Km

1.8Km

1.15Km

C2

35.7km

C4

C5

C8

C7

C6

Area C cell size:

x2=                                  =                   =

Region A total area

Minimum cell number

9182Km2

4.17

2199.63Km2

x=46.9Km              RB=46.9/2=33.16Km

Comparing the radius calculated by maximum base station parameters with this radius in part 1: the latter is smaller, so I select the radius according to capacity.

In the region C, the traffic is designed in macrocell, the minimum number of BS is 4.17, but there seems no way to design 4.17cells, I design 8 BSs.

In fact, here the planned side length of cell is 35.7km,

Offered traffic in C1,C3,C6,C8 cells:

T=t*x2=0.02*35.7^2=25.49  Erlang          37 TCs/5TRSs /BS giving a capacity of

28.25Erlang/cell

Offered traffic in C2,C4,C5,C7 cells:

T=t*x2=0.02*35.7*28.6=20.42  Erlang          29 TCs/4TRSs /BS giving a capacity of

21.04Erlang/cell

## The capacity planning in region A for Micro cell

1Km

1Km

A2

A3

A4

A1

A5

A6

A7

A8

A9

A10

A11

A12

A13

A14

A15

A16

Area A cell size:

16Km2

Region A total area

1.57Km2

x2=                                  =                   =

Minimum cell number

10.19

x=1.25Km              RA=1.25/2=0.89Km

Comparing the radius calculated by maximum base station parameters with this radius in part 1: the latter is larger, so I select the radius according to maximum base station parameters.

I design 16 BSs. The length of cell side x=1km

In fact, offered traffic in every cell: x=1km,           R=0.71km

T=t*x2=18*1^2=18  Erlang          29 TCs/4TRSs /BS giving a capacity of

21.04Erlang/cell

Summary of capacity planning

| region | T | Ncell | NBS | R/km | NTCH | Timpl |
| --- | --- | --- | --- | --- | --- | --- |
| A_macro | 45 | 1.61 | 4 | 2.23 | 88 | 59.6 |
| B | 84 | 2.87 | 4 | 10.1 | 116 | 84.1 |
|  | 81 |  |  |  |  |  |
| C | 145 | 4.17 | 8 | 25.24 | 264 | 197.16 |
|  | 118 |  |  |  |  |  |
| A_micro | 288 | 10.1 | 16 | 0.71 | 464 | 336.64 |

# Coverage Planning

According to service probability and the some other system parameter and the compared cell radius by capacity and max system parameter to calculate the required transmitted power level, antenna gain, eventual use of mast top amplifier, antenna height and DL/UL power level balance.

The parameters for every region

|  | Region A (macrocell) | Region B | Region C | Region A(microcell) |
| --- | --- | --- | --- | --- |
| Feeder diameter ¼ “……2_1/4” | 256dB7km…26.3dB/km | 256dB7km…26.3dB/km | 256dB7km…26.3dB/km | 256dB7km…26.3dB/km |
| Mast top amplifier | Gmat:10dB Fmat:2.0dB | Gmat:10dB Fmat:2.0dB | Gmat:10dB Fmat:2.0dB | Gmat:10dB Fmat:2.0dB |
| BSAntenna height | 30m…120m | 30m…120m | 30m…120m | 10m |
| MSAntenna height | 1.6m | 1.6m | 1.6m | 1.6m |
| BS sensitivity | -102dBm | -102dBm | -102dBm | -102dBm |
| MS sensitivity | -100dBm | -100dBm | -100dBm | -100dBm |
| BS max output power level | <46dBm | <46dBm | <46dBm | <46dBm |
| MS output power level | 30dBm | 30dBm | 30dBm | 30dBm |
| The combining filter loss | 3,0dBm | 3,0dBm | 3,0dBm | 3,0dBm |
| BS connector & jumper loss | 1,0dBm | 1,0dBm | 1,0dBm | 1,0dBm |
| The BS antenna gain | 21dBi | 21dBi | 21dBi | 15dBi |
| The MS antenna gain(including feeder and duplex loss | 2dBi | 2dBi | 2dBi | 2dBi |
| BS diversity gain | 5dB | 5dB | 5dB | 5dB |
| cell radius | 2.23km | 10.1km | 25.24km | 0.71km |

## Area A (macro cell)

### Downlink

The cell radius R=d=2.23km to calculate the path loss:

Lpath=46.3+33.9log(f)-13.8log(hbs)-A(hms)+(44.9-6.55log(hbs))log(R)+ Cm

=151.18dBm

Sms-Gbs+Ac+1.0+Lbsf+Lpath-Gms+SMFPbstx

Principle: when needed tx power level exceed available level, improvement is tried in the following order: higher antenna gain, larger feeder diameter, insertion of mast top amplifier in UL, larger BS antenna height

Here                   Gbs=21dBi

Gms=2dBi

feeder φ =3/8”

hbs=30m

Pbstx   42.02dBm      let Pbstx=43dBm                   Pmsrx= -99.02dBm

### Uplink

R=d=2.23km to calculate the path loss:

Lpath=46.3+33.9log(f)-13.8log(hbs)-A(hms)+(44.9-6.55log(hbs))log(R)+ Cm

=151.18dBm

Sbs - Gms+Lpath+SMF -Gbs+Lbsf+1-Gmtaeff -GdivPmstx

Here using      Gbs=21dBi

Gms=2dBi

feeder φ =7/8”

hbs=30m

Gdiv=5dB

without using mast top amplifier

Pmstx   28.13dBm      let Pmstx=29dBm                Pbsrx=  -101.13dBm

## Area B

Using the same method as in the region A  (macro cell) to calculate the needed tx power level.

### Downlink

R=d=10.1km

Lpath=46.3+33.9log(f)-13.8log(hbs)-A(hms)+(44.9-6.55log(hbs))log(R)

=159.35dBm

Here                 Gbs=21dBi

Gms=2dBi

feeder φ =2_1/4”

hbs=30m

Pbstx   45dBm              let Pbstx=45.5dBm                   Pmsrx= -99.5dBm

### Uplink

R=d=10.1km

Lpath=46.3+33.9log(f)-13.8log(hbs)-A(hms)+(44.9-6.55log(hbs))log(R)+ Cm

=159.35dBm

Sbs - Gms+Lpath+SMF -Gbs+Lbsf+1-Gmtaeff -Gdiv<=Pmstx

Here                   Gbs=21dBi

Gms=2dBi

feeder φ =7/8”

hbs=30m

Gdiv=5dB

Gmtaeff=7.47dB

Pmstx   28.82dBm               let Pmstx=29dBm                Pbsrx= -101.82dBm

## Area C

Using the same method as in the region A  (macro cell ) to calculate the needed tx power level.

### Downlink

R=d=25.24km

Lpath=46.3+33.9log(f)-13.8log(hbs)-A(hms)+(44.9-6.55log(hbs))log(R)

=153.32dBm

Here                 Gbs=21dBi

Gms=2dBi

feeder φ =3/8”

hbs=30m

Pbstx   44.16dBm               let Pbstx=45dBm                   Pmsrx= -99.16dBm

### Uplink

R=d=25.24km

Lpath=46.3+33.9log(f)-13.8log(hbs)-A(hms)+(44.9-6.55log(hbs))log(R)+ Cm

=153.32dBm

Sbs - Gms+Lpath+SMF -Gbs+Lbsf+1-Gmtaeff -Gdiv<=Pmstx

Here                   Gbs=21dBi

Gms=2dBi

feeder φ =2-1/4”

hbs=30m

Gdiv=5dB

Pmstx   28.97dBm             let Pmstx=29dBm                Pbsrx= -101.97dBm

## Area A (micro cell)

### Downlink

R=d=0.71km

Lpath=L0+Lrts+Lmsd=150.76dBm

Sms-Gbs−max+Ac+1.0+Lbsf+Lpath-Gms−max+SMF  <Pbstx

Here                 Gbs=15dBi

Gms=2dBi

feeder φ =1/4”

hbs=10m

Pbstx   43.53dBm              let Pbstx=45dBm                   Pmsrx= -98.53dBm

### Uplink

R=d=0.71km

Lpath=L0+Lrts+Lmsd=150.76dBm

Sbs - Gms+Lpath+SMF -Gbs+Lbsf+1-Gmtaeff -Gdiv<=Pmstx

Here                   Gbs=15dBi

Gms=2dBi

feeder φ =2-1/4”

hbs=10m

Gmtaeff=6.46dB

Pmstx   29.31dBm      let Pmstx=29.5dBm                Pbsrx= -101.81dBm

Summary of coverage planning

|  | Region A (macro)DL | Region A (macro)UL | Region B DL | Region B UL |
| --- | --- | --- | --- | --- |
| Feeder diameter ¼ “……2_1/4” | 3/8” | 7/8” | 2-1/4” | 7/8” |
| Mast top amplifier |  | Gmat:10dB Fmat:2.0dB |  | Gmat:10dB Fmat:2.0dB |
| BSAntenna height | 30m | 30m | 30m | 30m |
| MSAntenna height | 1.6m | 1.6m | 1.6m | 1.6m |
| BS sensitivity |  | -102dBm |  | -102dBm |
| MS sensitivity | -100dBm |  | -100dBm |  |
| The combining filter loss | 3,0dBm |  | 3,0dBm |  |
| BS connector & jumper loss | 1,0dBm | 1,0dBm | 1,0dBm | 1,0dBm |
| The BS antenna gain | 21dBi | 21dBi | 21dBi | 21dBi |
| The MS antenna gain(including feeder and duplex loss | 2dBi | 2dBi | 2dBi | 2dBi |
| BS diversity gain |  | 5dB |  | 5dB |
| feeder loss | 6.24dBm | 2.35dBm | 1,052dBm | 2.35dBm |
| Effective Gmtaeff |  |  |  | 7.47dBm |
| SMF | 3.6dBm | 3.6dBm | 3.6dBm | 3.6dBm |
| Cell radius | 2.23km | 2.23km | 10.1km | 10.1km |
| Average path loss | 151.18dBm | 151.18dBm | 159,35dBm | 159,35dBm |
| BS output power level | 43dBm |  | 45.5dBm |  |
| MS received power level | -99.02dBm |  | -99.5dBm |  |
| MS output power level |  | 29dBm |  | 29dBm |
| MS received power level |  | -101.13dBm |  | -101.82dBm |
| Power level unbalance | 0.11dBm |  | 0.32dBm |  |

|  | Region C DL | Region C UL | Region A (micro cell) DL | Region A (micro cell) UL |
| --- | --- | --- | --- | --- |
| Feeder diameter ¼ “……2_1/4” | 3/8” | 2-1/4” | 1/4” | 2-1/4” |
| Mast top amplifier |  | Gmat:10dB Fmat:2.0dB |  | Gmat:10dB Fmat:2.0dB |
| BSAntenna height | 30m | 30m | 10m | 10m |
| MSAntenna height | 1.6m | 1.6m | 1.6m | 1.6m |
| BS sensitivity |  | -102dBm |  | -102dBm |
| MS sensitivity | -100dBm |  | -100dBm |  |
| The combining filter loss | 3,0dBm |  | 3,0dBm |  |
| BS connector & jumper loss | 1,0dBm | 1,0dBm | 1,0dBm | 1,0dBm |
| The BS antenna gain | 21dBi | 21dBi | 21dBi | 21dBi |
| The MS antenna gain(including feeder and duplex loss | 2dBi | 2dBi | 2dBi | 2dBi |
| BS diversity gain |  | 5dB |  |  |
| feeder loss | 6.24dBm | 1.05dBm | 3,07dBm | 0.32dBm |
| Effective Gmtaeff |  |  |  | 6.46dBm |
| SMF | 3.6dBm | 3.6dBm | 2.7dBm | 2.7dBm |
| Cell radius | 25.24km | 25.24km | 0.71km | 0.71km |
| Average path loss | 153.32dBm | 153.32dBm | 150,76dBm | 150,76dBm |
| BS output power level | 45dBm |  | 45dBm |  |
| MS received power level | -99.16dBm |  | -98.53dBm |  |
| MS output power level |  | 29dBm |  | 29.5dBm |
| MS received power level |  | -101.97dBm |  | -101.81dBm |
| Power level unbalance | 0.81dBm |  | 1.28dBm |  |

# Frequency Planning

According to CCI and interference outage probability to calculate the possible frequency reuse factor to save the frequency resource.

Parameters for the frequency plan

| Co_channel protection ratio | 9dB |
| --- | --- |
| Voice activity factor | 0.4 |
| Channel activity factor | (1-B) Tcell |
| Interference outage probability P(CIR>PR) | 10% |

The average BS tx –power level (according to coverage planning)

| Area | the average tx –power level (DL) | the average tx –power level (UL) |
| --- | --- | --- |
| A(macro cell) | 43dBm | 29dBm |
| B | 45.5dBm | 29dBm |
| C | 45dBm | 29dBm |
| A(micro cell) | 45dBm | 29.5dBm |

BS equipment and auxiliary parameters

| Subregion | A(macro) | B | C | A(micro) |
| --- | --- | --- | --- | --- |
| Pbs(dBm) | 43 | 45.5 | 45 | 45 |
| α(dB/km) | 156 | 26.3 | 156 | 256 |
| hbs(m) | 30 | 30 | 30 | 10 |
| Lf=α(hbs+ 10)+Lfix | 10.24 | 5.05 | 10.24 | 7.07 |
| Gbs(dBi) | 21 | 21 | 21 | 15 |
| (macro):L0-a (hms)= 156.65- a (hms) -13.8lg(hbs) | 135.94 | 124 | 103.96 |  |
| n=4.49-0.655lg(hbs) | 3.52 | 3.52 | 3.52 | n=2.0+kd=4.8 |
| Log(R)(km) | 0.35 | 1 | 1.4 | -0.15 |

PRX=PR-ΔPtx+ΔLf-ΔGbs+ΔL0+10ΔnlogRc+10logηcarr+10logηdtx

The necessary PRX value (dB)

| PRX | A(macro) | B | C | A(micro) |
| --- | --- | --- | --- | --- |
| A(macro) | 3.24 | 22.78 | 37.22 | -24.01 |
| B | -16.09 | 3.54 | 17.89 | -51.73 |
| C | -31.2 | -10.59 | 3.76 | -70.96 |
|  | -30.44 | -10.82 | 3.54 | -71.18 |
| A(micro) | 28.23 | 47.86 | 62.21 | 3.54 |

Normalized minimum reuse distance for interference into different sub- region

| NI | 1 | 2 | 6 |
| --- | --- | --- | --- |
| Sub-region A (macro) (cell radius R=2.23km) |  |  |  |
| A | 1.95 | 2.48 | 3.4 |
| Sub-region B (cell radius R=10.1km) |  |  |  |
| B | 1.98 | 2.5 | 3.42 |
| Sub-region C (cell radius R=25.24km) |  |  |  |
| ###### C | 2 | 2.51 | 3.48 |
|  | 1.98 | 2.5 | 3.42 |
| Sub-region A(micro) (cell radius R=0.71km) |  |  |  |
| A | 1.7 | 2.51 | 3.42 |

Significance of co-channel interference between the subregions A(macro) and C

|  | A1 | A2 | A3 | A4 | C1 | C2 | C3 | C4 | C5 | C6 | C7 | C8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A1 | X | X | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| A2 | X | X | 0 | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| A3 | X | 0 | X | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| A4 | 0 | X | X | X | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| C1 | X | X | X | X | X | X | 0 | X | 0 | 0 | 0 | 0 |
| C2 | X | X | X | X | X | X | X | X | X | 0 | 0 | 0 |
| C3 | X | X | X | X | 0 | X | X | 0 | X | 0 | 0 | 0 |
| C4 | X | X | X | X | X | X | 0 | X | 0 | X | X | 0 |
| C5 | X | X | X | X | 0 | X | X | 0 | X | 0 | X | X |
| C6 | X | X | X | X | 0 | 0 | 0 | X | 0 | X | X | X |
| C7 | X | X | X | X | 0 | 0 | 0 | X | X | X | X | X |
| C8 | X | X | X | X | 0 | 0 | 0 | 0 | X | 0 | X | X |

For micro cell in region A, in effect when I consider 2 or more interference, can’t reuse frequency highly. When I consider only one interference, use 8 group frequencies:

Table of frequency plan

| A(macro cell) | The number of carrier frequency |
| --- | --- |
| A1,A3 | 1,3,5 |
| A2,A4 | 2,4,6 |
| C1,C8 | 7,9,11,13,15 |
| C2,C7 | 17,19,21,23 |
| C3,C6 | 8,10,12,14,16 |
| C4,C5 | 18,20,22,24 |
| A(micro) |  |
| A1,A11 | 25,27,29,31 |
| A2 ,A12 | 33,35,37,39 |
| A3, A9 | 26,28,30,32 |
| A4,A10 | 34,36,38,40 |
| A5,A15 | 41,43,45,47, |
| A6,A16 | 49,51,53,55 |
| A7,A13 | 42,44,46,48 |
| A8,A14 | 50,52,54,56 |
| B1,B4 | 57,59,61,63 |
| B2,B3 | 58,60,62,64 |