Drlink - Digital radio link route design program

Short Guide

0. Installation

Copy/extract the files drlink32.exe, drlink32.lbr and dr32.hlp into the same directory, e.g. C:\drlink. Make normal shortcuts or program groups according to your will.

1. General

To run Drlink you need a personal computer with Microsoft Windows 95  or

Windows NT (3.51 or higher).  You need the files drlink32.exe and

drlink32.lbr to run the program, and these files must be in the same directory.

The library (drlink32.lbr) may have several releases to be used with the same

version of the main program. You may check the current library version from

the about box via help menu. The version number is also printed on the

calculated hop sheets.

With Drlink you can design a single hop or a route consisting of several hops

in the frequency range 1.5 ... 57 GHz. You select radio, antenna and feeder

types, give antenna heights, performance criteria and Drlink optimizes the

design according to antenna and feeder prices. Or you give all data manually

and let Drlink calculate the performance. You can save results on the disk

and modify the files later and print the results to a printer or a file.

2. Program structure

The program consists of three pages:

* RADIO		the selection of the radio

* BASICS		input of basic parameters

* FORM		calculation

To switch from a page to another select a command from the Pages-menu or

use keyboard shortcuts ^R, ^B, or ^F.

The program saves two kinds of files:

* single hops		filename.hop

* routefiles		filename.rou

3. Hop calculation

When the program is started,  you can choose from four options :

* ONE HOP             		Calculate a new single hop

* MODIFY OLD HOP	Modify existing single hop

* NEW ROUTE		Design a new route

* MODIFY OLD ROUTE	Modify existing route

To calculate a new hop, press the 'One hop' - button to get to the Radiopage.

You design a hop using the three pages of Drlink : Radiopage,

Basicpage and Formpage. Usually, you go through the pages in order

starting from Radiopage but you can move freely between pages to

change values.

On Radiopage you select the radio for the current hop. You also

specify the transmitter output power (Ptx), receiver threshold power

level (Prxth) and bit error rate (BER). On the left side of the Radiopage, a

list of available radios is shown. The information about the current radio is

shown on the page.  The desired transmission power and receiver threshold

power are selected using the buttons.

If you want to use a special radio not shown in the list box in your

calculations, choose the OTHER item in the list box. Drlink displays a

dialog box where it asks the parameters for your radio. However, these

values cannot be saved to a file.

If you want to cancel the selections you made, choose the Reset

button. This reselects the values that were valid when you entered

Radiopage.

When you have completed the radio parameter selection, you can

choose the Basics command from the Pages Menu, and  Drlink will

display Basicpage.

On Basicpage you select antenna and feedertypes, calculation

formulas, and type other basic information used in calculations.

St 1/2, reference height:  Type here the names of the stations and the

heights of the station reference levels in meters above mean sea level

(amsl). The actual heights (amsl) should be used because the program

uses the link path height (whether below or above 700 m) in selecting

the calculation coefficients. The station reference levels are found by

fitting the linear regression line to the path profile (minimum mean

square fit, excluding the first and last 1-2 kilometers of the hop) and

finding the heigths of this line at station locations. See ITU

documentation (Rec 530-5 or -6, 1994, 1995) or corresponding other

documentation for further details. You may want to use for this job

Prof**.XLW or equivalent Excel-file, which gives also other useful

info regarding hop design.

Outage objective:  Type here the outage objective of your hop.  If you

choose to optimize the design on Formpage, Drlink uses this objective

as a criteria. If you do not use the optimizing feature, this value is of no

importance. Note that this regards the SES objective (and multipath

fading) only. For unavailability there is no place to put in.

Latitude and longitude: Type here the approximate location of the

hop using the general coordinates, e.g., 60 (degrees) N(orth),

22 (degrees)  E(east).

Time percentage: Type here the time percentage that the average

refractivity gradient in the lowest 100 m of atmosphere is less than -100

N units/km. This is given by the ITU charts 7-10  in Rec. ITU-R PN.453-4.

Consult Rec 530 or other related documentation for further details.

ITU formula: You can choose between two formulas, ITU Rec 530-6 (1995)

Method 1 or Method 2. Method 2, the more accurate and preferred

one, is the default option.

Land type: Select here the predominant land type under the link from the list.

Selective fading: You have two choices to give selective fading margin

on Formpage:

* Standard	you type the effective fading margin for a hop length

of 42.5km (Rummler's model) and Drlink scales the

value to the current hop length.

* User		you type the effective fading margin for the current

hop length.

Note: normally no calculations regarding selective fading are needed

with 4-state modulated (QPSK, CPM e.t.c.) radios below capacities of

about 20 Mb/s.

Modes of diversity and usage: Select the diversity and mode of usage

for the hop. Drlink will propose the antenna branching attenuation on

the Formpage according to the selections. (Check on the Formpage

that the value is correct and change if necessary.)

When you have completed this page, choose the Form command

from the Pages Menu, and Drlink will display Formpage.

On Formpage you calculate the performance of the hop. You type the

feeder lengths and antenna heights (calculated from the station reference

heights) on this page. Then you can either give the antenna diameters

and feeder thicknesses yourself and calculate the resulting outage, or

you can let Drlink calculate the most economical design (Optimize)

according to the prices of antennas and feeders.

The antenna diameters can be chosen from the library values by

pressing the spin buttons. Spin buttons are the buttons which look like

arrows.The gain is calculated automatically. The feeder

attenuation/100 m is likewise chosen using the spin buttons. The offered

antenna and feeder types depend on the selections made on the

Basicpage.

You can give your own antenna diameters and feeder attenuations by

typing a value in the box. Note that the respective antenna gain is not

calculated automatically. You can give the gain manually, or, if you

wish, you can let Drlink calculate the gain of your antenna by leaving

the gain box empty. (Note: not by typing 0.0 in the box.) In this case

the gain is calculated when you select the Calculate button.

The antenna heights must be given in meters above the station

reference level (asrl) given on the Basicpage. This need not be the

same as the actual height from the ground (although it sometimes

may be quite close to that value). Currently there is no place for the

actual tower heights to put in as these are not required in the

calculations.

Drlink proposes the antenna branching loss according to the diversity

and usage combination selected on Basicpage. If there is no value in

the library for the selected combination, Drlink displays 0.0 in the edit

box and you must type the correct value yourself.

When you want Drlink to calculate the error performance using the

values you chose, select the Calculate button or press the return key.

Drlink checks the values you typed on the Formpage and, if there were

no errors, displays the results in a separate Results window.

With Drlink you can calculate the most economical design that fulfils

the performance criteria (SES percentage). Drlink uses the prices of

antennas and feeders from the library file when calculating and

comparing different combinations. When you select the Optimize

button, Drlink calculates different antenna diameter and feeder

thickness combinations, compares the price and error performance

and displays 15 cheapest combinatios that fulfil the criteria (if that

many combinations exist). In addition 5 combinations that fall outside

the criteria are shown at the end of the list. These combinations are

marked with a star. Select the desired combination by pressing the Ok-

button. The results-window will be displayed. This optimization gives only

a rough starting point for the design as the prices vary depending on the

manufacturer,  time and other factors. These variations cannot be

taken into account by the software.

The annual unavailability due to rain can be calculated by pressing the

RainCalc button. The rain attenuation calculation is possible when the

radio frequency is greater than 6.0 GHz. Because the fading margin is

needed in the rain calculation, the selecting of RainCalc button

causes first the calculation of the error performance (as when

Calculate button is selected). The worst month percentage is

calculated for additional info only and it will not be printed. The

designer should be aware that the SES percentage and unavailability

should be kept separated as these both are used to specify the total

link quality.

You can select the polarization used and you should give the local rainrate

exceeded 0.01% of the time. The default rainrate is 22 mm/h. Note that

when calculating the effective hop length and the given local rainrate

exceeds 100 mm/h, the value 100 mm/h is used instead. This is

according to the CCIR recommendation 530 (1992, 1994, 1995).

4. Route calculation

With Drlink you can design a route, which consists of one or more hops. First,

you define the route parameters and hops belonging to the route. Drlink

creates route and hop files. Then you can start designing the hops one by

one. You may also make a route from existing hop files.

You define the route parameters, and name the hops that belong to the route

in the RouteScreen dialog box .

Radio: Select the radio common for all hops in the route. If needed, you can

change this default radio when calculating a hop.

Route directory: Here you must type the directory where the route and hop

files will be saved. It is strongly recommended to create an own subdirectory

for each route in order to avoid overwriting of a hop file belonging to a

different route. The route directory must be created separately, as Drlink

cannot create a subdirectory.

Route filename: Drlink will save the route data under this file name. The

default extension, which Drlink appends automatically unless you type an

extension of your own, is .ROU. The routefile will be created when you press

the HopCalc button. If the file exists, it will be overwritten.

Route outage objective: Type the route objective to this edit field. Note that

this is the SES percentage.

Outage division method: You have three choices to divide the route objective

between the routes:

* by number 	the objective is divided equally between hops

* by length	the objective is divided between hops in the ratio

of their length

* manually	you type the objective of each hop yourself; the sum

of hop objectives should equal to the route objective

By pressing the 'HopDef' - button you can define the hops belonging to

the route. You can also load data from previously calculated hops. The

hop must be in the route directory. Note that the outage objective of

the hop is taken from the division method. If some other division

method than manual is selected, the original objective of the old hop

will be overwritten with the new value. By pressing the 'RouteSC'-button

you can return to the RouteScreen-window.

When all the hops belonging to the route have been defined, the hops

may be calculated one by one by pressing 'HopCalc'-button. Use the

View Hops-command in the File-menu to move from one hop to

another.

5. Saving and printing

Saving and printing is possible on every page by selecting the

respective command from the File-menu.

The routefile cannot be saved manually. It will be written

automatically when calculating hops belonging to the route.

When saving, the default filename extension for hopfiles is HOP and for

route files ROU. The extension will be added automatically to the

filename, unless the filename already has an extension or the filename

ends with a period.

You can print a hop or route document on the current printer or you can save

the document as an ASCII file on the disk. You can print any time during

calculation, and the values not yet defined will be displayed as 0.0. There are

two modes in printing to file, with and without delimiters. The former form

may be useful if the results are transferred to other programs, like Excel for

instance.

6. Error messages

Drlink checks the data you type in edit fields every time you try to move to

another page, before calculation and saving. If needed, Drlink puts up a

message. There are messages of three types :

Invalid data: a given value is of wrong type. For example there may be letters

among numbers. The erroneus value must be corrected before proceeding.

Value over validity limits: Values (either given or calculated) are outside the

validity of the used formulas.

Here are the validity limits for either given or calculated values and of which a

warning is given. Depending on the variable and limit in question, the

calculation is stopped or just a warning is given.

Basicpage:

- longitude           			( 0 .. 180) (stopped)

- latitude               			( 0 .. 90) (stopped)

- hop length            			< 0.1 (stopped)

Formpage:

- antenna heights (asrl)		< 0.0 (stopped)

- grazing angle				< 1 mrad ( warning)

- grazing angle				< 0.2 mrad (stopped)

- fading margin			< 12 dB (warning)

- fading margin			< 0.0 (stopped)

- diversity improvement		> 100 (warning)

Value over normal limits: a given value exceeds usual limits. The program

confirms if you want to continue. This kind of warning will come only once

during one session for each variable. Regardless of whether you selected the

Ok or the Cancel button, Drlink will not warn about the variable again.

Here is a list of the variables for which the warning over normal limits may

occur, and the limits for each variable:

Basicpage:

- outage objective   			( 0.0005 .. 0.02 )

- hop length             			( 0.2   .. 100.0 )

- time percentage pL                	( 0.5 .. 80)

Formpage:

- antenna branching loss  		( 1.0    .. 12.0 )

- additional terrain loss		( 0.0    .. 15.0 )

- threshold degradation		( 0.0    .. 5.0 )

- diversity ant. spacing		( 3.0    .. 23.0 )

- freq. div. channel spacing 		( fd/f < 5% )

- selective fading margin   		(20.0    .. 60.0 )

- feeder lengths                 		( 5.0    .. 200.0 )

- rain rate				(20    .. 200)

IPR, Feedback and bug reporting

This program has been developed under the supervision of NOKIA Research

Center and reflects the current general understanding (according to ITU and

others) on radio link hop calculations.

The software is distributed within Nokia and to our customers

without any guarantee or liability ("as such").

The customer may make necessary backup copies for his/her own use.

Limited distribution within one company and in one country only is

allowed provided that the full package including at least drlink32.exe,

drlink32.lbr and this file are given at the same time and unchanged.

If other distribution is required, if you find errors or have ideas to develop

this further you may contact the nearest NOKIA representative.

NOTE: This is a demo version and the radio parameters may differ from the actual ones. This should not be used for any real design.

No part of this publication or software may be copied, distributed, transmitted,

transcribed, stored in a retrieval network, or translated into any human or

computer language without the prior written permission of Nokia

Telecommunications Oy (except the cases above). The manufacturer has

made every effort to ensure that the instructions contained in the documents

are adequate and free of errors and omissions. The manufacturer's liability for any errors in the documents is limited to the correction of errors and advisory services. The manufacturer  has no liability whatsoever as regards the software.

The documents and software have been prepared to be used by professional

and properly trained personnel, and the customer assumes full responsibility

when using them. The manufacturer welcomes customer comments as part of

the process of continual development and improvement of the documentation

and software in the best way possible from the user's viewpoint. Please

submit your comments to the nearest Nokia sales representative.

NOKIA is a registered trademark of Nokia Corporation.

Helsinki, November 1997

Copyright © Nokia Telecommunications Oy 1997. All rights reserved.

Copyright © Nokia Research Center 1997. All rights reserved.