## S-72.227 Digital Communication Systems

### Spring 2002	Home Work#3, Deadline February 18,2002 at 10.00

Suppose a binary FSK signal with discontinuous phase is generated by switching between two oscillators with outputs Accos(2πfot+θo) and Accos(2πf1t+θ1). Since the oscillators are unsynchronized, the FSK signal may be viewed as the interleaved sum of two independent binary ASK signals. Use this approach to find, sketch, and label Gc(f) for f > 0 when fo =fc-rb/2 and f1=fc+rb/2 with fc >> rb.