## S-72.227 Digital Communication Systems

### Spring 2002	Solution for Homework#9, 29.04.2002

A CDMA system consists of 15 equal-power users that transmit information at a rate of 10kbps, each using a DS spread spectrum signal operating at a chip rate of 1 MHz. The modulation is binary PSK.

  - Determine the Eb/Jo, where Jo is the spectral density of the combined interference.

  - How much should the processing gain be increased to allow for doubling the number of users without affecting the output SNR?

Thus,WRJavPav=WRNu−1=EbJ0⇒EbJ0=WR1Nu−1=106104115−1=7.143⇔8.539dBANSWER:

a) We have 15 users transmitting at a rate of 10kbps each, in a bandwidth of 1MHz. Thus, Nu=15, W=1MHz and R=10kbps

- With Nu=30, and Eb/Jo=7.143, the processing gain should be increased by W/R=7.143*(30-1)=207 time more.

Hence, the bandwidth must be increased to 207MHz