E 11.1

## S-72.227 Digital Communication Systems

### Spring 2002	Tutorial#11, 29.04.2002

An optical source is selected from a batch characterized as having lifetimes which follow a slow internal degradation mode. The –3dB mean time to failure of these devices at room temperature is specified as 5x104 h. If the device emits 1 mW at room temperature, what is the expected optical output power after 1 moth of operation? After 1 year? After 5 years?

###### E 11.2

Consider a sinusoidally modulated optical signal P(t) of frequency , modulation index m, and average power Po given by

P(t)=Po(1+mcosωt)2

show that, when this optical signal falls on a photodetector, the mean square signal current  is2> generated consists of a dc (average) component and IP and a signal current ip given by

is2>=Ip2+<ip2>=RoPo2+12mRoPo2

where the responsivity Rois given by

R=IpPo=ηqhν

E 11.3

An engineer has the following components available:

  - GaAlAs laser diode operating at 850 nm and capable of coupling 1 mW (0 dBm) into a fiber.

  - Ten sections of cable each of which is 500 m long, has a 4 dB/km attenuation, and has connectors on both ends.

  - Connector loss of 2 dB/connector

  - A pin photodiode receiver

  - An avalanche photodiode receiver

Using these components, the engineer wishes to construct a 5-km operating at 20 Mb/s. If the sensitivity of a pin and APD receivers are –45 and –56 dBm, respectively, which should be used if a 6-dB system operating margin is required.