- S-72.245 Transmission Methods in Telecommunication Systems

- Tutorial 10

- Objectives

- To investigate coherent and incoherent binary systems and respective error rate analysis

- To identify the respective circuit block diagrams and signaling waveforms

- Quizzes

- Q10.1

Sketch signal constellations for  a) 8-PAM 	b) 8-QAM 	c) 8-PSK

- Q10.2

Explain by your own words what is a signal constellation diagram?

- Q10.3

Symbol error ratio for 8-PSK in AWGN channel can be determined by

.

Decision signal is characterized by Ac=1 and noise rms-value of 97.7 dBV. What is the respective symbol error rate?

- Matlab assignments

M10.1 Design a digital implementation of the transmitter and receiver filters GT(f) and GR(f) such that their product satisfies

GT(f)GR(f) = Xrc(f)

where Xrc(f) is the raised-cosine frequency response characteristic enabling the ISI at the sampling time t=nT to be zero. GR(f) is the matched filter to GT(f).

Hints:

- 1) 	Raised-cosine frequency response Xrc(f):

Xrc(f)=T,0≤f≤1−α2TT21+cosπTαf−1−α2T,1−α2T<f≤1+α2T

where 1/T is the symbol rate.

- 2)	The simplest way to design and implement the transmitter and receiver filters in digital form is to employ FIR filters with linear phase (symmetric impulse response).

M10.2 Consider a channel-distorted pulse x(t, at the input to the equalizer, given by the expression

x(t)=11+(2tT)2

where 1/T is the symbol rate. The pulse is sampled at the rate 2/T and is equalized by a zero-forcing equalizer. Determine the coefficients of a five-tap zero forcing equalizer.